TPS 100

LSS Data Output

Three options are supplied for LSS Data Output.

LSS

This option outputs an LSS Load Input File as observed data.

LSS_C

This option outputs an LSS Load Input File as X, Y, Z Co-ordinates.

LSS_RAW

This option outputs an LSS Load Input File as observed data but without a Station Co-ordinate record.  [i.e. Without record 2,]

Job

The Set Job Program [PROG 1] enables the input of the Survey and Surveyor Names.  It is used by the Data Output to create elements of the Header for the LSS file so should always be used at the start of a job.

It should always be the first operation carried out at the beginning of a new job.

Station and Orientation

The Set Station Program [PROG 2] should be used for each new Instrument Set Up.

The program will use the computed bearing from the Instrument Station co-ordinates to the RO Station co-ordinates, or the input Angle, to set the orientation of the Instrument.

This bearing or input angle is recorded in the SET-UP DETAILS record [Code 3] so that the recorded Horizontal Angles to the measured points are correct.

[User feedback on this aspect would be appreciated].

Measurement

Measurement uses Info1 to place the Codes into the appropriate measurement record and Info2 to add a String Number if required.  This enables the user to have "sensible" prompts in the Code Field of the Code List as shown in the example, LSS.CDE.

The Codes can be entered by using the Numeric Key Pad or by pressing the CODE key.

The first ten Codes in the Code List are automatically assigned to the keys in the Numeric Key Pad.  When the DSP key is pressed until COD : is displayed on the second line of the display, pressing the appropriate numeric key triggers a measurement [as if the ALL key was pressed] and automatically records the Code.  If the Code List has a ? prompt in one of the fields the Instrument will go to the first one and await the user input in the normal way.

When creating a Code List it is important to have the first ten lines with your most frequently used codes to take advantage of this feature.  The example Code List, LSS.CDE, shows how a Code List can be constructed.

Free Cod makes Info1 available for Codes only whilst Free + S makes Info1 available for the Code and Info2 available to add a String Number.  For example; if the number 2 is entered against the String ? prompt for Top of Bank prompt the recorded code would be TOB2.

The Code must always be recorded before the Measurement.

Typical Usage

Use MENU, 3.DATA MANAGER, 5.DELETE ALL, 1.OBSERV to clear the Observation data memory to start a new job.

Use PROG, 1.Set Job to enter the Job details.  This must be the first record in the new data set and is used to create the Header for the LSS file.

Use PROG, 2.Set Station to enter the Station and Orientation information for the Station Set Up.

Measure and Record your detail points.

To Measure and Record a point with a Code either use the Numeric Key Pad if the Code is one of the first ten in the Code List, or use CODE and ALL.  Note that each time a Numeric Key is used both the Measurement and Code are recorded.  To maximise the number of points that can be stored only Code when necessary.  For example; to measure Spot Heights using the example Code List LSS.CDE use Numeric Key 0 to measure, record and code the first point then ALL for the subsequent points.

Example Data

LSS Data Output from TPS100 using LSS.FRT

0, Survey

1, DMS, VASD

0, Job: LSS_JOB Surveyor: SURVEYOR Instrument: TC605 409693

0, 14:19 10-06-1997

2, STN1,   1000.000,   1000.000,     10.000

3, STN1, STN2,       0, 1.500

5,  0128,  495542,  803726,    6.177, 1.222, SH

5,  0129,  571206,  865357,    6.292, 1.222, SH

5,  0130,  621510,  795038,    5.753, 1.222, SH

5,  0131,  871412,  803721,    6.174, 1.222, TOB3

5,  0132,  993450,  795041,    5.751, 1.222, TOB3

5,  0133,  943049,  865357,    6.291, 1.222, BOB1

9

LSS Data Output from TPS100 using LSS_C.FRT

0, Survey

0, Job: LSS_JOB Surveyor: SURVEYOR Instrument: TC605 409693

0, Co-ordinate Data File 14:19 10-06-1997

21,  0128,   1004.664,   1003.923,     11.284, SH

21,  0129,   1005.281,   1003.403,     10.618, SH

21,  0130,   1005.011,   1002.636,     11.292, SH

21,  0131,   1006.084,   1000.294,     11.284, TOB3

21,  0132,   1005.582,    999.058,     11.292, TOB3

21,  0133,   1006.262,    999.506,     10.618, BOB1

9

LSS Data Output from TPS100 using LSS_RAW.FRT

0, Survey

1, DMS, VASD

0, Job: LSS_JOB Surveyor: SURVEYOR Instrument: TC605 409693

0, 14:19 10-06-1997

3, STN1, STN2,       0, 1.500

5,  0128,  495542,  803726,    6.177, 1.222, SH

5,  0129,  571206,  865357,    6.292, 1.222, SH

5,  0130,  621510,  795038,    5.753, 1.222, SH

5,  0131,  871412,  803721,    6.174, 1.222, TOB3

5,  0132,  993450,  795041,    5.751, 1.222, TOB3

5,  0133,  943049,  865357,    6.291, 1.222, BOB1

9

*****   TPS 100 Data File   *****

Job   Surveyor      Inst.     Number    Time         Date

LSS_JOB   SURVEYOR      TC605     409693   14:19   10-06-1997

Station         Height       Eastings      Northings

STN1         10.000       1000.000       1000.000

STN2      000 00'00"      1000.000       1100.000

Code      Info 1      Info 2      Info 3      Info 4      Info 5

+ Spot Ht   +      SH   +           +           +           +

Point No     Hz Angle      V Angle  Slope Dist   Rf Ht

128   049 55'42"   080 37'25"       6.177   1.222

129   057 12'06"   086 53'57"       6.292   1.222

130   062 15'10"   079 50'38"       5.753   1.222

Code      Info 1      Info 2      Info 3      Info 4      Info 5

+Bank Top   +     TOB   +       3   +           +           +

Point No     Hz Angle      V Angle  Slope Dist   Rf Ht

131   087 14'12"   080 37'20"       6.174   1.222

132   099 34'49"   079 50'41"       5.751   1.222

Code      Info 1      Info 2      Info 3      Info 4      Info 5

+Bank Bot   +     BOB   +       1   +           +           +

Point No     Hz Angle      V Angle  Slope Dist   Rf Ht

133   094 30'49"   086 53'57"       6.291   1.222

*****   End of TPS 100 Data File   *****

Leica GSI format for Comparison

410001+0Spot Ht 42....+000000SH 43....+00000000 44....+00000000 45....+00000000 46....+00000000

110002+00000128 21.024+04955420 22.004+08037260 31...0+00006177 51..0.+0000+000 87..00+00001222 88..00+00001500

110003+00000129 21.024+05712060 22.004+08653570 31...0+00006292 51..0.+0000+000 87..00+00001222 88..00+00001500

110004+00000130 21.024+06215100 22.004+07950380 31...0+00005753 51..0.+0000+000 87..00+00001222 88..00+00001500

410005+Bank Top 42....+00000TOB 43....+00000003 44....+00000000 45....+00000000 46....+00000000

110006+00000131 21.024+08714120 22.004+08037210 31...0+00006174 51..0.+0000+000 87..00+00001222 88..00+00001500

110007+00000132 21.024+09934500 22.004+07950410 31...0+00005751 51..0.+0000+000 87..00+00001222 88..00+00001500

410008+Bank Bot 42....+00000BOB 43....+00000001 44....+00000000 45....+00000000 46....+00000000

110009+00000133 21.024+09430490 22.004+08653570 31...0+00006291 51..0.+0000+000 87..00+00001222 88..00+00001500

Leica UK Limited, Davy Avenue, Knowlhill, Milton Keynes, England, MK5 8LB.