# Use of code lists with TPS1100

There are two files contained in the Code list folder

- A default code list ‘.CRF’ created by LSS from the prototype survey. This needs to be copied onto the instrument PCMCIA card ‘CODE’ folder and then use the instrument menu option Code List management to select it from the available list.

  - Note: If a user defined code list is required then the option in LSS ‘Export / Upload Code Lists to Survey Instruments’ produces an IDX file which can be imported to Leica Geo Office Code List Manager, then edited if necessary, and Saved as a ‘CRF’.

- A parameter file ‘.PAR’ contains the instrument configuration settings, including both display and record masks that will automatically configure the instrument for LSS.

  - First copy this file to the \TPS\CONF folder.

  - From the instrument menu select “Configuration/Load”

  - Then “Load System Parameter”.

  - Use “Load Configuration” and Load the parameter file

Some important menu settings made by this parameter file are the Quick Code settings in the Configuration menu under “Instrument Configuration\Point and measure parameters”

    - Quick code – REC before ALL

    - Quick code – With 2 digits