Leica TPS1100 Series

Using QSET with LSS

Using QSET to record the Occupied and RO Station Co-ordinates, Instrument and Target Heights, for a Station Set Up for use with LSS.

The Meas Job and Data job files must be one and the same so that all the QSET data is recorded in the same file for use with LSS.

Using QSET

F5	SETUP

F4	QSET

Station Id:	Enter the Occupied Station Identifier

Stn.East  :	Enter the Eastings of the Occupied Station

Stn.North :	Enter the Northings of the Occupied Station

Stn.Elev  :	Enter the Height of the Occupied Station

F3	REC		To record the Co-ordinates for the Occupied Station

Backs.  Id:	Enter the Orientation Station Identifier

Back.East :	Enter the Eastings of the Orientation Station

Back.North:	Enter the Northings of the Orientation Station

Back.Elev :	Enter the Height of the Orientation Station

F3	REC		To record the Co-ordinates for the Orientation Station

Inst. Ht. :	Enter the Height of Instrument

Refl. Ht. :	Enter the Height of the Target Reflector

F2	DIST		To Measure the Distance to the RO as a check *

F4	CONT		To proceed to the Set Station screen

F3	REC		To Set and Record the Station information

* 	Once the Instrument Height and Reflector [Target] Height have been entered, the screen shows the prompt “Measure to Backs.point”.

Either press

F2	DIST		To Measure the Distance to the RO as a check

F4	CONT		To proceed to the Set Station screen

Or

F4	CONT		To proceed to the Set Station screen

Do not press ALL [F1] or REC [F3] while this prompt is displayed or you will record data before the Instrument circle has been set to the correct Orientation.

Using FreeStation/Resection with LSS

The Freestation or Resection options both automatically adjust the horizontal circle to the calculated bearing value after the resection observations themselves have been recorded. This means that they will be related to the previous set-up in the converted LSS load file unless one is specifically coded. The number of these observations is 2 for the Resection and between 2 and 10 in the Freestation option.

The instrument reports the resection calculation by creating a station coordinate record in the GSI file that also contains a '25' word of the circle adjustment which LSS uses to warn the user that a new set-up record may be necessary in the load file.

LSS configuration settings

In LSS, select Configure / Hardware and System / New Logger

Logger format - Leica

Leica data type - LSS Standard

Port settings are not needed as Leica Survey Office is chosen for the communications

This configuration is only needed once on an installed version of LSS

LSS conversion of Leica GSI data to LSS load file format

In LSS, select Input / Download and Convert

Select logger previously configured with Configure / Hardware and System / New Logger as above

Follow prompts for running Leica Survey Office and downloading data file.